3D Reconstruction Rasmus:

I tested the 3D triangulation by reconstructing 6 sample images taken from Matt & Luka videos. To see the results use the code reconstruct_3D_images.m. Type reconstruct_3D_images(1) to see image1, reconstruct_3D_images(2) to see image 2 and so on.

To manually label more images use extract_3D_images.m.

3D calibration Rasmus:

Here we provide the scripts to calibrate the cameras for triangulating body landmarks.

Cameras are calibrated with known real-world objects (Lego® plates). To map 2D coordinates to the 3D known coordinates we first take pictures of the Lego® plates from each of the four cameras. In each Lego® plate the top of each brick is painted with fluorescent markers. To assign each brick to its known 3D coordinates use the script detect_lego_markers_simple_rp.m. Instructions on how to run this interactive script are provided in CalibrationInstructions_RP.ppt.

For estimating camera matrices use the script calibrate_cameras_rp.m. This script use the direct linear method (script DLT_simple_cl.m see help therein) to estimate each camera matrix P separately.  Triangulation is then obtained via LS linear triangulation (script ls_triangulate.m) that can be applied to an arbitrary number of views. The script calibrate_cameras_rp.m also generates a figure showing nominal and estimated 3D coordinates of Lego® plates.

Note Rasmus:

Camera Names: There are four cameras, called 093 (camera1), 385 (camera2), 683 (camera3), 907 (camera4).