3D Reconstruction Rasmus:

I tested the 3D triangulation by reconstructing 6 sample images taken from Matt & Luka videos. To see the results use the code reconstruct_3D_images.m. Type reconstruct_3D_images(1) to see image1, reconstruct_3D_images(2) to see image 2 and so on.

To manually label more images use extract_3D_images.m.

3D calibration Rasmus:

Here we provide the scripts to calibrate the cameras for triangulating body landmarks.

Cameras are calibrated with known real-world objects (Lego® plates). To map 2D coordinates to the 3D known coordinates we first take pictures of the Lego® plates from each of the four cameras. In each Lego® plate the top of each brick is painted with fluorescent markers. To assign each brick to its known 3D coordinates use the script detect_lego_markers_simple_rp.m. Instructions on how to run this interactive script are provided in CalibrationInstructions_RP.ppt.

For estimating camera matrices use the script calibrate_cameras_rp.m. This script use the direct linear method (script DLT_simple_cl.m see help therein) to estimate each camera matrix P separately.  Triangulation is then obtained via LS linear triangulation (script ls_triangulate.m) that can be applied to an arbitrary number of views. The script calibrate_cameras_rp.m also generates a figure showing nominal and estimated 3D coordinates of Lego® plates.

Note Rasmus:

Camera Names: There are four cameras, called 093 (camera1), 385 (camera2), 683 (camera3), 907 (camera4).

Segment Videos:

To find parts of the videos where the mouse is visible first use:

track_mouse_box_rp('camera1_cut', 'camera1_bg', false, false)

[This function generates 3 outputs. The first two (idx and idy) are the x and y coordinates of the body centre for each frame. The last one, bright_th, is the average brightness (normalized) of the 1st darkest percentile of each frame. When is very negative (e.g. -0.5) then the 1st darkest percentile is very dark meaning that the  animal (which is dark) is visible. When is less negative (e.g. -0.05) then most of the animal is not visible.]

Then load results:

load('camera1_cut_track.mat')

Finally type

TH = -0.5;

frame_id = segment_tracks(bright_th, TH);

The output is a cell array e.g.

frame_id: {1×82 double}    {1×68 double}    {1×7 double}    {1×79 double}    {1×46 double}    {1×75 double}

Each cell contains the index of all the consecutive frames for in which the animal is visible. By changing the value of the threshold TH the results will change. Specifically for a highly negative threshold (e.g. TH= - 0.7) there will be fewer frames in which the animal is visible. For a low negative  threshold (e.g. TH= - 0.1) there will be many frames in which the animal is deemed visible.